7-1 Final Project: Thermostat Lab

The plan to build a smart thermostat for SysTec can be developed using one of three hardware architectures: Raspberry Pi, Microchip, or Freescale.

A Raspberry Pi is a beginner-friendly microcontroller board that is cost-effective, while a Microchip is an industrial-grade, heavy-duty chip used for secure commercial products and a Freescale is a traditional, high-performance microcontroller used heavily in medical equipment and automotive systems.

The Raspberry Pi has two dedicated internal lanes called I2C controllers that use SDA for data and SCL for clock to talk directly to the AHT20 without conflict. It has 26 GPIO pins and uses built-in PWM channels to easily make LEDs pulse and fade smoothly. It also uses built-in electronic resistors inside the chip, so there is no need for extra hardware resistors to stop button signals from bouncing. The LCD screen has sufficient digital pins to plug in the multiple wires needed for the 16x2 display screen.

The Microchip features built-in I2C hardware blocks designed with industrial noise filtering, which helps ensure it receives accurate temperature data from the sensor. It has up to 60 high-performance GPIO pins and uses built-in Output Compare hardware to smoothly pulse LEDs. It also uses an instant hardware interrupt system that detects button presses immediately without slowing down the system. Since it has many pins, the LCD screen can easily connect using its parallel data wires.

The Freescale uses standard I2C modules to communicate with the sensor through direct memory access, which can grab temperature readings and store them in memory automatically without using the main CPU. It uses a highly configurable multiplexer to assign roles to its pins, allowing LEDs to connect to FlexTimer blocks that execute precise hardware fade cycles. Buttons use built-in debounce filters to ensure clean single presses. It handles the LCD screen using a flexible digital layout block called FlexIO, which can be programmed to function as a custom parallel interface for the character display.

Wi-Fi cloud connectivity varies between the Raspberry Pi, Microchip, and Freescale architectures. The Raspberry Pi has a small wireless co-processor chip built onto the board next to the main processor. The main chip communicates with the wireless chip over an internal data bridge, allowing it to connect to standard home or office Wi-Fi networks. It can package temperature and operating mode information into a data string and send it directly to a server. The Microchip has Wi-Fi integrated into the main chip, allowing it to send data without additional wiring. It also includes a built-in Trust&Go security engine that helps it connect securely to enterprise cloud servers. Freescale-based designs may require a separate Wi-Fi module to provide cloud connectivity. In that case, the main processor communicates with the external Wi-Fi hardware, which forwards thermostat data to a web server.

For memory usage, the Raspberry Pi provides 2 MB of Flash memory and 264 KB of RAM. The Microchip provides up to 2 MB of Flash memory and 640 KB of RAM. The Freescale provides approximately 1 MB of Flash memory and 256 KB of RAM. All three architectures provide enough memory to support the thermostat code, temperature monitoring, display functions, button inputs, and cloud communication features.

Overall, the Raspberry Pi should be recommended for SysTec's smart thermostat. It supports all of the required peripherals, provides built-in Wi-Fi connectivity for cloud communication, and has enough Flash and RAM to support the thermostat software. In addition, it is beginner-friendly and cost-effective, making it a practical choice for SysTec's smart thermostat project.

References:

Microchip Technology Inc. (2021). PIC32MZ W1 and WFI32E01 Family Data Sheet. Microchip Technical Documentation.

NXP Semiconductors. (2014). Kinetis K64F Sub-Family Data Sheet. Freescale Semiconductor Technical Data.

Raspberry Pi Ltd. (2022). Raspberry Pi Pico W Datasheet. Raspberry Pi Documentation.